1/   op1:                       	max: 	LODL 1	; op2

op2:                            		SUBL 2	; op2-op1

opres: 0                        		JNEG op1big:

.LOC 50                     		LODL 1

main: 	LODD op1:                	 	RETN  	; op2

PUSH              	op1big:	LODL 2

LODD op2:                 		RETN  	; op1

PUSH

CALL max:

INSP 2

STOD opres:

HALT

2/ main: LODD index

DESP 2	; sp:=sp-2;

LOCO 0	; ac:=0

STOL 0	; m[sp+0]:=ac (sum:=0)

STOL 1	; m[sp+1]:=ac (i:=0)

5: 	SUBL 2	; ac:=ac-m[sp+3](i-N)

JZER 15	; jump if i==N

JPOS 15	; jump if (i>N)

LODL 0	; ac:=m[sp+0] (sum)

ADDL 1	; ac:=ac + m[sp+1] (sum+i)

STOL 0	; m[sp+0]:=ac (sum=sum+i)

LOCO 1	; ac:=1

ADDL 1	; ac:=ac+m[sp+1](1+i)

STOL 1	; m[sp+1]:=ac (i=1+i)

JUMP 5	;

15: 	LODL 0	; ac:=m[sp+0] (sum)

INSI 2	; sp:=sp+2 move stack pointer down 2 places

STOL 1	; m[sp+1]:=sum(return sum)

RETN	; return to caller

3/ 	STOD result: 	; store ac to result location

4/ adder: 	LODL 1    ; get 1st arg from stack into ac (data count)

STOD mycnt	; store count at location mycnt:

LODL 2 ; get 2nd arg from stack into ac (data addr)

PSHI		; push indirect 1st data to stack

ADDD myc1:	; add 1 (value at myc1:) to addr in ac

STOD myptr:	; store new addr to location myptr:

5/ MySub: 	LODL 2	; ac:=topNum

SUBL 3	; ac:=ac-bottomNum

PUSH

JNEG neg:

LODL 2	; ac:=result

POPI 		; m[ac]=m[sp]

sp is at local var (value of subtraction),

gan gia tri subtraction into result

LOCO 0	; ac:=0

RETN

neg: 	LODL 2

POPI

LODD cn1:	; ac:=m(x); x:address of cn1 & value=-1

RETN

cn1: 	-1

6/ Recursion

LOOP: LODD PasCnt	; num of fibs to in PasCnt

JZER DONE:	; no more passes, go to DONE

SUBD c1:

STOD PasCnt:	; --pass remaining

P1:   	LODD daddr:	; load a pointer to fib arg

PSHI       	; push arg for fib on stack

ADDD c1:

STOD daddr 	; inc, store pointer for next d[n]

CALL FIB:  	; call fib (arg on stack)

INSP 1     	; clear stack on fib return

P2:   	PUSH       	; put return AC(fib(n)) on stack

LODD faddr:	; load a pointer to result f[n]

POPI       	; pop result off stack into f[n]

ADDD c1:

STOD faddr:	; inc, store pointer for next f[n]

JUMP LOOP:	; go to top for next pass

FIB:  	LODL 1     	; fib func loads arg from stack

JZER FIBZER:	; if fib(0) go to FIBZER

SUBD c1:   	; dec arg value in AC (arg-1)

;;;;Iterative;;;

STOD LpCnt:	; num of iterations in LpCnt

LODD c0:   	; load a 0 into AC

STOD fm2:  	; store 0 in fib(n-2)

LODD c1:   	; load a 1 into AC

STOD fm1:  	; store 1 in fib(n-1)

ITER: 	LODD LpCnt:	; LpCnt arg-1 iterations needed

JZER RTN:  	; when LpCnt==0 go to RTN:

SUBD c1:   	; dec arg value in AC (LpCnt-1)

STOD LpCnt:	; store LpCnt for next iteration

LODD fm2:  	; arg must be >= 2, fm2 initially fib(0)

ADDD fm1:  	; fm1 initially fib(1), AC=fm2+fm1

STOD tmp:  	; store AC to tmp:

LODD fm1:  	; now load AC with fib(n-1)

STOD fm2:  	; replace old fib(n-2) with AC

LODD tmp:  	; load AC with tmp: becomes fib(n-1)

STOD fm1:  	; store AC as next fib(n-1) to fm1

JUMP ITER:

RTN:  LODD tmp:  	; load AC with tmp: final result

RETN

;;;;Recursion;;;

PUSH 	; pushing (arg-1)

CALL FIB:	; fib(arg-1)

PUSH

LODL 1	; load value(arg-1) to AC

SUBD c1:	; arg-1 is decremented

PUSH	; push(arg-2)

CALL FIB:	; call fib(arg-2)

INSP 1	; remove (arg-2) from stack

ADDL 0	; ac=f1+fm2

INSP 2	; remove f1 & (arg-1) from stack

RETN

;;;;

FIBZER: 	LODD CO:

RETN	; ac=0 for fib(0)

FIBONE: 	LODD c1:

RETN	; ac=1 for fib(1)

1/ Convert base 10 into base 2-8-16:

Base 2: 119.78125: 119/2↑.78125*2↓ => 01110111.11001

Base 8: group 3 digits: 001|110|111|.|110|010| => 167.62

Base 16: group 4 digits: 0111|0111.1100|1000| => 77.C8

2’s complement = flip each single bit + 1

| 18.687510 = ?2 18/2 = 9 => 0↑ 9/2 = 4 => 1↑ 4/2 = 2 => 0↑ 2/2 = 1 => 0↑ 1/2 = 0 => 1↑ => 1810 = 100102 | .687510 .6875*2=1.375 => 1↓ .375*2=0.75 => 0↓ .75*2=1.5 => 1↓ .5*2=1.0 => 1↓ => .687510 = .10112 => 18.687510 = 10010.10112 |
| --- | --- |

2/ Convert 2’s complement into base 10:  -1: 1111   -2: 1110   -3: 1101   -4: 0100   -5: 0011   -6: 0010   -7: 0001   -8: 1000   +7:0111   +6:0110   +5: 0101   +4: 0100   +3: 0011   +2: 0010    +1: 0001

Sign bit: Positive: 0 Negative: 1

Magnitude: Flip all digits included sign bit to get magnitude

11011011 => ⌐00100100 + 1 = 001001012 = 37 => Negative: -37

Positive: 0110|10012 = 6916 = 6*16 + 9 = 10510

3/ Convert ** into floating number (base 10):

** IBM format: 1 (Sign)|7 (Exponent)|24 (Mantissa)   bits

Exponent: base 16, excess 64.    0 ≤ Mant. no hidden bit < 1

0:+|100 0011:67|0101 1100 0000 0000 0000 0000

Exp.=67-64=3=>163=212=> result =212*(2-2+2-4+2-5+2-6)

= 210 + 28 + 27 + 26 = 1472.0

** IEEE 754 single precision:

1 (Sign)|8 (Exponent)|23 (Mantissa) bits

Base 2 excess 127    Hidden bit = 1; = 0 if exponent is all 0’s

0:+|1000 0110:134| 101 1100 0000 0000 0000 0000

134-127=7 => 27*(1+2-1+2-3+2-4+2-5) = 27+ 26+24+23+22 = 220.0

Ex.: Exponent: 0000 0000: 2-126

1 1111 1111   .0110 = …0000 => NAN

1 1111 1111   .0000 = …0000 => -∞

4/ Convert floating number into IEEE 754 single precision:

.812510        .1101    Move the decimal point to L: +, R: -

*          2        1.F           Move 1 => 127 – 1 = 12610

1.6250↓                    => Exponent: 011111102

=> 0|0111 1110|101 0000 0000 0000 0000

5/ Add 2 IEEE 754 single precision:

Fl.1: 0|1000 0010|101 1100 0000 0000 0000 0000 => 13.75

Fl.2: 0|0111 1110|101 0000 0000 0000 0000 0000 => 0.8125

Exponent Float 1: 27 + 21 = 128 + 2 = 130 - 127 = 3

Float 2:                               = 126 - 127 = -1

- shift 4 bits of mantissa 2

Mantissa 1: HB1 	101 1100 0000 0000 0000 0000

+ Mantissa 1: HB0 	000 1101 0000 0000 0000 0000

110 1001 0000 0000 0000 0000

Result: 0|1000 0010|110 1001 0000 0000 0000 0000

HB1

6/ 01100101.00111:

IEEE 754 shifts 6 places, +6 =>  01.10010100111

IBM shifts 8 places, +2         =>  .0110010100111

IEEE 754: 127+6=133: 	1000 0101|  100 1010 0111 0000…

IBM        : 64+2=66    :        	0100 0010|0110 0101 0011 1000…

7/ Most current machines use 32-bit words. Limit addresses to 4GB, hi-end systems use 64 -bit words. Address space = 1.8*1019 bytes.

8/ Byte ordering: Conventions

Ex: 1521310 = 3B6D16 int: 4 bits

| 6D | => | 00 |
| --- | --- | --- |
| 3B |  | 00 |
| 00 |  | 3B |
| 00 |  | 6D |

- Big endian: least significant byte has highest (numerically largest) address. Little endian: least sig. has lowest address

0x100    0x101    0x102    0x103

Big E: 	01  |  23  |  45  |  67  |

Little E: 	67  |  45  |  23  |  10  |

-String: char s[6]=”18243”. Character “0” has code 0x30. Digit i has code 0x30+i. Final character=0

-Unsigned values: UMin=0, UMax=2W-1 (-1 b/c of sign bit)

2’s complement: TMin=-2W-1, TMax=2W-1-1

-Sign Extension: make k copies of sign bit

-IEEE 754:

Least Positive/N normalized: 0/1 0000 0001 0000…0000 = ±2-126 =~ ±1038

Most 	0/1 1111 1110 1111…1111 = ±2128  =~ ±1038

Least Pos./Neg. denorm’ed:	0/1 0000 0000 0000…0000 = ±2-149 =~ ±10-45

Zero	0     0000 0000 0000…0000 = 0

Pos/Neg Infinity	0/1 1111 1111 0000…0000 = ±∞

NAN	0/1 1111 1111 ≠0…..       ≠0 = NAN

9/ a) base 10 value if it is a signed 2’s complemented int?

1 111 111 110 010 101 => 2+8+32+64+1 => -107

0 000 000 001 001 101 => 1+4+8+64 => +77

1 111 111 110 010 101

0 000 000 001 001 101 (+)

1 111 111 111 100 010 => 1+4+8+16+1 => -30

Example 1: Suppose that IEEE-754 32-bit floating-point representation pattern is 0|10000000|110 0000 0000 0000 0000 0000

Sign bit S = 0 ⇒ positive number

E = 1000 0000B = 128D (in normalized form)

Fraction is 1.11B (with an implicit leading 1) = 1 + 2-1 + 2-2 = 1.7510

The number is +1.75 × 2128-127 = +3.510

Example 2: Suppose that IEEE-754 32-bit floating-point representation pattern is 1|01111110|100 0000 0000 0000 0000 0000

Sign bit S = 1 ⇒ negative number

E = 0111 1110B = 126D (in normalized form)

Fraction is 1.1B (with an implicit leading 1) = 1 + 2-1 = 1.5D

The number is -1.5 × 2126-127 = -0.75D

Example 3: Suppose that IEEE-754 32-bit floating-point representation pattern is 1|01111110|000 0000 0000 0000 0000 0001

Sign bit S = 1 ⇒ negative number

E = 0111 111010 = 12610 (in normalized form)

Fraction is 1.000 0000 0000 0000 0000 0001B  (with an implicit leading 1) = 1 + 2-23

The number is -(1 + 2-23) × 2126-127 = -0.500000059604644775390625 (may not be exact in decimal!)

Example 4 (De-Normalized Form): Suppose that IEEE-754 32-bit floating-point representation pattern is 1|00000000|000 0000 0000 0000 0000 0001

Sign bit S = 1 ⇒ negative number

E = 0 (in de-normalized form)

Fraction is 0.000 0000 0000 0000 0000 0001B (with an implicit leading 0) = 2-23

The number is -2-23 × 2-126 = -2×-149 ≈ -1.4×10-45

Example 5: -176.375 the sign is negative. 176.375=27 + 25 + 24 + 2-2 + 2-3

Move 7 bits to the L, => 10110000.0112 = 1.01100000112*27

Calculate exponent = 127+7=134 = 100001102

Result: -176.375=1|10000110|011 0000 0110 0000 0000 00002

Example 6: 107=1101011‬2    .21875=.001112‬‬‬‬‬‬‬‬‬‬‬‬

=> 107.21875=1101011.001112 => shift left 6 places

=> 1.101011001112*26     Exponent = 127+6=133=10000101‬2 ‬‬‬‬‬‬‬‬‬‬‬‬

Result in IEEE754: 107.21875=0|10000101|1010110011100000…2

=> 107.21875=1101011.001112 => shift left 8 places => +2

=> 0.01101011001112 => exponent = 64+2=66=010000102

Result in IBM: 107.21875=0|1000010

Subref: 	LODL 1;

PSHI;

LODL 3

PSHI

LODL 1

SUBL 0

INSP 2

RETN

Denormalized floating point …=> Tiny

Memory address space on the Mic1 is the best => 16-bit addressable with 2 to the 12th

EC7A=> FFFF EC7A repeat the sign bit

0        9          5        C       7       6        9        4

Address 0x095C7694=> 0000 1001 0/101 1100 0111 0110/1001 0100‬‬‬‬‬‬‬‬‬‬‬

12                      5C76

Tag = 0x12    Set # = 0x5C76

Tag           | Set # 	| Block offset

16 bits	    12 bits          4 bits

4 Hex	    3 Hex	        1 Hex

Direct-mapped cache with a 16-byte (24 => offset=4) block size and 4K (=212 => set # = 12) sets.

E8|79F|0	 miss		11101000||011110011111||0000‬‬‬‬‬‬‬‬‬

E8|79F|4	 hit		11101000||011110011111||0100‬‬‬‬‬‬‬‬‬

A4|79F|8 miss		10100100||011110011111||1000‬‬‬‬‬‬‬‬‬

E8|79F|8 miss (conflict)	11101000||011110011111||1000‬‬‬‬‬‬‬‬‬

A4|79F|C miss (conflict	10100100||011110011111||1100‬‬‬‬‬‬‬‬‬

Tavrg seek = 7ms

Tavrg rotation = ½ * (60 sec/10,000rpm) * (1000ms/sec) = 3(ms)

Tavrg transfer = (60 sec/10,000rpm)*(1/1,000sects/track)*1,000ms/sec = 0.006ms

Taccess = 7 + 3 + .006 = 10.006ms

Taccess = Tavrg seek + Tavrg rotation + Tavrg transfer

Tavrg rotation = ½ * 1/RPMs * 60sec/1min

Tavrg transfer = 1/RPM * 1/(avr # sectors/track) * 60 sec/1min

Example: Given Rotation rate =7200rpm, avr seek time = 0ms, avg # sector/track = 400.

Derived:

Tavrg rotation = ½ * (60secs/7,200rpm)*1,000ms/sec=4ms.

Tavrg transfer = 60/7,200rpm*1/400 sects/track * 1,000ms/sec = 0.02ms.

Taccess = 9ms+4ms+0.02ms

Begins at line 100

100: mar := sp; rd;

101: rd;

102: a := mbr;

103: b := a;

104: b := a + b;

105: b := b + b;

106: b := a + b;

107: mbr := b; wr;

108: wr; goto 0;

** IBM format: 1 (Sign)|7 (Exponent)|24 (Mantissa)   bits

Exponent: base 16, excess 64.    0 ≤ Mant. no hidden bit < 1

0:+|100 0011:67|0101 1100 0000 0000 0000 0000

Exp.=67-64=3=>163=212=> result =212*(2-2+2-4+2-5+2-6)

= 210 + 28 + 27 + 26 = 1472.0

** IEEE 754 single precision:

1 (Sign)|8 (Exponent)|23 (Mantissa) bits

Base 2 excess 127    Hidden bit = 1; = 0 if exponent is all 0’s

0:+|1000 0110:134| 101 1100 0000 0000 0000 0000

134-127=7 => 27*(1+2-1+2-3+2-4+2-5) = 27+ 26+24+23+22 = 220.0

lodl 1; get address of Num1

pshi ; push value of num onto stack

lodl 3; get address of num2

pshi ; push value of num2 onto stack

lodl 0; AC = num2;

addl 1; AC=num2 + num1;

insp 2; clean up stack for two values pushes

retn